GENERAL USER MANUAL

FOREWORD

Thank you for your purchasing motorcycle.

This  manual  covers  the  main  data,basic  structure,and  main  procedures  of  operation,  adjustment,maintenance and troubleshooting of motorcycle.It will help you familiarize yourself with all the necessary skills so that you will bring your vehicle into full,best play with minimized trouble for a long service life.

Products  are  always  subject  to  further  improvement,which  will  cause  some  difference between the vehicle and this manual,without further notice.

CONTENTS

I  .Safe Drive  	 1

Ⅱ.Main Data  	3

Ⅲ. Parts and Subassemblies     	 5

IV.OPERATION   	9

Ignition  Switch  	9

Fuel Cook 	 9

Engine  Starting  	10

Switches  on   Right Handlebar     	11

Switches  on  Left Handlebar    	12

Gear Shifting   	13

V. Check -ups ,Adjustments  and  Maintenance     	14

Machine  Oil Checking  	14

Renewal  ofMachine   oil  	14

Cleaning  of Machine  Oil  Tank 	15

Check -up of Spark  Plug  	15

Check -up , Cleaning  of Air Filter  	16

Adjustment  of ThrottleCable     	17

Adjustment  of Carburetor   	17

Check -up  & Adjustment  of AirValve Gap  	18

Adjustment  of Clutch  	20

Brake Checking   	 20

Adjustment  of Front Brake 	 21

Adjustment  of Rear Brake  	22

Adjustment  of Chain  	 22

Adjustment  of Braking Light Switch 	23

Battery  hecking   	 24

Replacement   of Fuse  	 24

Vehicl Washing 	25

Maintenance  in Non-use Time 	 26

Resumption  of Service  	 27

Maintenance  Routine  Diagram 	28

VI.Electrical   diagram  	30

I.SAFE DRIVE

Rules for Safe Drive

Check must be conducted,before starting theengine,to prevent mishaps and damage to components.

Only the qualified person,who has passed the drive examination and to whom a drive license has been issued, is permitted to drive the vehicle but not anybody else without a drive license.

Full preoccupationis required during drive,paying attention to the following points to avoid any possible hurt to you by other motorized vehicles:

Donot drive too close to lther vehicles; Never contend for lane.

Strictly observi the local traffic rules.

As driving at overspeed is the cause of many accidents,do not drive at a speed the actual situation does not permit.

Turn on the turnlight when making a turn orchanging the lane.

Particular care should be exercised at the level crossing of roads,entance and exit of parking lot or on the automobile lane.

Duringdrive,grasp the left handlebar by the left hand and the throttle twist grip by the right one,with feet on the footrests.

The luggage carrier is designed for carrying light goods,which should be securely fastened to prevent loose movement that may cause mishaps during drive.

1

Protective Wear

1.Protective wear such ashelmet with protective mask,dustproof glasses and gloves shoule be worn duringdrive forthe sake of personal safety.

2.The passenger should wearhight boots or long clothes to protect legs from hurt by the heated exhaust silencer during ride.

3.Loose clothes are not suitable for motorcycledrive or ride as they may get caught on the operating lever,kick lever,footrestor wheel,resulting in danger.

Modification of the vehicle

Caution:

Any unauthorized modification of the vehicleorreplacement of the originalparts cannot ensure driving safety and is illicit.The user must observe the regulations ofthe traffic controlauthorities. We are not responsible for any vehicle with unauthorized modification.

Loading of goods

Caution:

The design of the motorcycle requires distribution of the carried goods in certain extent of

equilibrium  and  improper  arrangement  of goods  will  adversely  affect  the  performance  and

stability of the vehicle.The manufacturer shall not take any responsibility  due  to  the  reason mentioned  above.

# ILMAIN DATA

| Overall length | 2030mm | Cylinder bore ×stroke | 39×41.8mm |
| --- | --- | --- | --- |
| Overall width | 710mm | Compression ratio | (8.75~9.36):1 |
| Overall height | 1250mm | Output,max. | 2.4kW/7500r |
| Wheelbase | 1300mm | Torque,max | 3.2N.m/5500 |
| Dry weight | 96kg | Idlingspeed | 1500r/min |
| Max.load | 150kg(including the driver) | Displacement of cylinder | 49.3ml |
| Front wheel | 2.50-17 | Spark plug | A7RTC |
| Rear wheel | 2.75-17 | Spark plug gap | 0.6-0.7mm |
| Speed,Max | ≥45km/h | Cap of air Valve | Intake valve:0.03-0.07mm |
| Brake distance | ≤7m(30km/h |  | Exhause valve:0.03-0.07mm |
| Climbability | ≥10° |  |  |

ToBe Continued …

MAIN DATA

| Volure of lubricating oil | 0.9L | Fuse | 15A |
| --- | --- | --- | --- |
| Capacity of gasoine tank | 4.0L | Front light illuminator | 12V-35W/35W |
| Transmission ratio |  | Taillight/Braking light | 12V-5W/21W |
| Ist gear | 2.615 | Betraying light | 12V-5W |
| 2nd gear | 1.556 | Turnlight | 12V-10W |
| 3rd gear | 1.136 | Neutral light | 12V-3W |
| 4th gear | 0.917 | Turn indicatior | 12V-3W×2 |
| Transmission ratio of sprocket | 2.730 | Meter light | 12V-3W×2 |
| Primary transmission ratio | 4.059 | High beam indicator | 12V-3W |
| Battery | 12V5Ah |  |  |
| Ignition means | C.D.I |  |  |

4

# Ⅲ.PARTS&SUBASSEMBLIES

5

| No. | Name | Description |
| --- | --- | --- |
| ① | Speedometer | Indicate the speed of the motorcycle (Km/h). |
| ② | Turn indicator | The left light is on whenturning left,and the right light is on when turning right. |
| ③ | Gear position display | Indicate the gear position of the motorcycle |
| ④ | Odometer | Display the total mileage of the vehicle (Km) |
| ⑤ | Fuel leve indicator | Indicate the fuel level in the fuel tank |
| ⑥ | High beam indicator | It is lit up when the full beam is on |
| ⑦ | Neutral indicator | When in the neutral position,the letter of indicator light N lights up |
| ⑧ | Logo | When the instrument pane is turned on,the indicator lights up |
| ⑨ | Clock | When the meter is turned on,the clock will work |
| ⑩ | Photosensitive window |  |
| ⑪ | Tachometer | It shows the speed(rpm)of the engine. |
| ⑫ | The left button | The short technique to achieve ODO and TRIP,thestate of conversation:in ODO state,long press to achieve measurement and imperialchange,and in TRIP state,long press to achieve zero trave |
| ⑬ | The right button | Keep pressing until enteringthe clock setting mode,the clock flashes;In the clock setting mode,press briefly to achieve the current flashing bit+1;Press and hold to switch to the next flashing.When the minute flashes,press and save ,then quit the adjustment mode. |

6

8

# IV.OPERATION

## Ignition Switch

★Ignition swicth

| Position | Function | Remarks |
| --- | --- | --- |
| OFF | To stop thevehicle (swiching olf al circuits) | The key can be removed outtaker |
| ON | hirsuaring or driving the vehiclelmaikingall themaincicuits) | The key cam no beremoved |
| LOCK | To lockthe steering handle | The key can be taken out |

OFF

O

mm

IGNITION

sut

LOCK

ON

## Fuel   Cock

① Fuel filling

The capacity of the fueltank is 4.0L in total including 0.9L

of reserve.Leadless gasoline ofNo. 93 or above or low_lead gasoline  is gequired for the motorcycle.To  fuel the vehicle,support it by the  main stand,open the lock cover of  the fuel tank and fill fuel through  theopening,and thenclose the tank by the cover with the two on them  in good alignment.

② Operation  of  the  fuel cock(the valve of fuel tank)

ON:With the handle of the   fuel   cock   to“ON”position,the  fuel  circuit  is through  for  fuel supply.

OFF:With the handle of the  fuel  cock  to“OFF”position,the fuel  circuit  is  cut  off without

supply.

RES:With the handle of the  fuel cock to RES position,the fuel is supplied from the reserve.

(Note:The reserved fuel can  only be used when the normalsup-  ply is run out.)In this case,refuel-  ing should be carried out as soon  as possible,for there is only some 0.9L of fuel reserve for use.

Fuel  cockhandle

9

## Engine Starting

①Set the key of theignition switch to“ON”position.

②Set  the  emergency  stopswitch  to“O”position.

③Ascertain the neutral position,where it should be displayed.

④Ascertain the amount offuel in the tank.

⑤Set  the  fuel  cock  handle  to“ON”position. ★ To start a cold engine:

①Pull up the choke bar of the carburetor(to colse the  choke).

②Rotate the throttle twist grip by 1/8 to1/4 turn.

③Start the engine by the electric or the kick starting system.

④Slightly turn the throttle twist grip to increase the speed of the engine so as to warm up the engine.

⑤Turn the carburetorchoke bar downward to"B",fully open the choke when the engine is sufficiently warmed up.

★Caution:

The engine can only be started after the neutral position is ascertained.Otherwise   accident will happen.

Unnecessary idle running(especialy at a high speed)is harmful to the engine.

★Procedures of stopping engine:

①Release the throttle twist grip to slow down theengine.

②Turn to the neutral position.

③Set  the  ignition  switch  key  to“OFF” position.

④Set the fuel cock(the fuel tank valve)handle to“OFF”position.

## Switches      on      Right      Handlebar

①Headlight switch

The Headlight switch has three positions"&" 、"P<" and"·"(a  white  point).

"      ":When the switch is in this position,tail the headlight and meter lights are all lit up.

"P≤":When the  switch  is in this position,the tail, betraying and meter lights are lit up.

" ·":When  it  is  in  this  position,the  headlight  tail, betraying and meter lights are all off.

The headlight and taillight will be lit up only after the     vehicle     is     started.

②Electric start button(no such device for the vehicle of kick start mode only)

The  electric  start  button  is  located  below  the

headlight  switch.The  engine will be  started by Press- ing down this button.

③Emergency stop switch

When starting the engine,set the emergencystop switch  in"冈 "position todirectly stop the engine by cuttingoffthe electric power.

Thorttle twist grip

Start button

Front      illuminator      switch

## Switches       on       Left       Handlebar

①Light changing switch

D position,Headlight is in high beam.

D position,Headlight is in low beam.

②Turnlight switch

Position,Left

Position,Right

③Horn button

Press this button to horn.

## Gear Shift

Warm up the engine for normal runing.

①When  the  engine  is  idling,disengage  the clutch and tread the gear_shifting pedal to et the gear to thelst position.

② Gradually increase the speed of the engine    and slowly release the clutch lever,with a good coor- dination betweenthe two operations to ensure a natural   driving  start.

③ When the motorcycle reaches  a  balanced   state of running,slowdown the engine,disengage the  clutch again and tread the shifting pedal to change  the gear to the 2ndposition.Thegearcanbeshifted to other positions in the same way.

Shifting  forward

N

4

3

2

Shifting   backward4

13

## Machine    Oil    Checking

The    vehicle    should    be

checked for machine oil before  drive by supporting it with the main stand on a flat ground.The oil level  should be between the upper and  lower lines of theoil gauge,which  is  not  screwed  into  the  filling

# V.Check-ups,Adjustments         and          Maintenance

| Machine oil |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 20 |  | M.6 0 | 0 26 |  |  |
|  |  |  |  | 0 | 0 A3 |  |  |  |  |
| ℃ | -30 -20 -10.010203040 |  |  |  |  |  |  |  |  |
| F | -22 -414325068.86104 |  |  |  |  |  |  |  |  |

orifice.

High quality 4-stroke ma-

chine oil,as Class SEorSDin API classification,of SAE 15W-40QE in viscosity will help maintain a  long service life of the engine.In case those are not available,a sub-  stitute  suitable  for  the  ambient

Machine oil gauge

temperature of application should  be selected according to the table on the right side.

## Renewal       ofMachine       oil

Machine oil plays a very important role   in the normal operation of the engine and for  that reason,it is necessary to check the mo- torcycle for machine oil periodically and re- new the oil once every 800-1000 km of drive by the following procedures.

Remove the screw plug from the bottom of the hot engine to drain offall old oil.

Wash the oil filter screen clean and re-  mount it really to position.Then fill in 0.9L  fresh machine oil and start the enging for idle running 2-3 minutes..

Let the engine stop for 2-3 minutes,and  check to see whither the oil livel is in be- tween the upperand lower line the oil gauge.

Do not use any machine oil of a different    grade than the specified oneto avoid machinery failure.

## Cleaning    of    Machine    Oil    Tank

①Drain off all the run-in machine oil from the oil  tank.

②Dismount the related parts.

③Wash clean all the related parts.

④Fill in the required oil.

*This jobshould not be done by any untrained   persons but shall be done at an authorized service center

Screw plug for oil draining

0.6～0.7mm

## Check-up    of    Spark    Plug

①Remove the cap of spark plug  and screw off the spark plug by the plug wrench.

②Clean   the   spark   plug   all   around or replace it ifit is corroded or there is too much deposit on it.

③Regulate the gap of the spark plug to 0.6-0.7mm.

15

④The spark plug of the desig- nated type should be used.

## Check-up,Cleaning      of     Air      Filter

Take out the air filter and check ifit is contaminated. Dismounting:

Remove the left side cover screw of the filter,open the left cover and disassemble theair filter

Cleaning:

Wash the filterin clean washingoil and wipe itdry with dry cloth.

Soak the filterelement in clean machine oil,squeeze it dry and fit it back to position.

Recommended oil:15W/40QE

Caution:

The air filter  element for use must be  intact or the  engine  will  suck  in  dust  and  dirt,resultinging  a shorter  service  life  of the  engine.

Wather  should beprevented from  entering  into the filter  in  washing  the  vehicle.

The   filte   shall   nerver    loe   cleamed   with   gasolinge on any other agentof a lowignition point.

Collar clamp

## Adjustment of Throttle Cable

Make sure that theadjusting nut of the throttlecable works normally.

Check to see if the throttle twist grip is with the required free operating movement.

The required free operating movement:2-6mm.

Ifthe grip can not be so moved freely,turn the ad- justing nut to ensure it.

*After   adjustment,start   the   engine   and  check for the free operating movement again. Repeat the adjust ment if necessary until it is as required.

Adjusting nut              Locking nut

## Adjustment   of   Carburetor

Caution:

The idling speed adjustment of the engine should be carried out with a hot engine.

Set the idling speed to the required value   by the helpof the idling speed adjusting screw with the vehicle standing on a flat ground.

The required idlingspeed:1,500 r/min.

Idling speed adjusting screw

17

## Check-up &Adjustment of Air Valve Gap

Noise will stem from too big gap of the air valve.However if there is too small gap or even no gap at all,closing  of the valve will be hindered,which will cause burn of the valveand output drop.Therefore,the airvalve gap must be checked periodically.

The gap of the air valveshould be inspected and adjusted with a cold engine by the following procedures:

①Remove the caps of the central holeand the top hole(the ignition timing observation hole)in the left crank- case cover.

②Remove the caps of the two air valves on the cylinder head.

③Insert the "T"key into the central hole of the crankcase cover,jam itagainst ghe nut of the flying wheel and   then turn the flying wheel clockwise until the engraved "T"mark on the flying wheel aligns with the engraved line on the top of the crankcase cover.Swing the rocking arm slightly.A loose rocking arm(which indicates the existence of clearance)shows that the piston is in the lowerstop position of the compressing stroke.In this case,continuously turn the"T"key colckwise for 360degrees until the alignment of those engraved marks,where the valve can be adjusted.  Afterwards,check the valve gap by inserting a feeler in between the valve adjustingscrew and the end ofthe valve.

The specified air valve gap:0.03-0.07mm for the intake and exhaust valves respectively.

8

④ If the adjustment needed,loosen the locking nutof the valve,turn the adjusting nut till a slight resistance is felt on inserting the feeler

At theend of the adjustment,tighten the"Locking nut"to prevent loosening and conduct another check to make sure that the valve gap is OK before all those dismounted caps are refitted on.

Central hole cap

## Adjustment   of   Clutch

★The clutch should be adjusted with the engine in stopped state.

There should be a free operating move-   ment of 10-20mm at the endof the clutch le- ver as shown in the figure on the right side.

When adjustment is needed,loosen the    locking nut on the clutch operating cable and set the clutch lever to the required range of free operating movement.In case of adjust-   ment to be made to a great extent,turn the    clutch  adjusting  screw  studon  the  right  crankcase.

Is tart she engine to ascertain whether the adjusted clutch works normally.

★Readjustmenthas to be made ifthere   is slipping of clutch or difficulty in the en- gagement  ofgears.

## Rrake     Checking

(1)Pull up the front and rear brakes respectively and check   for wear of the brakeshoes.If the mark""on the brakedrum cover aligns with that""on the brake cam,it means that the brake

shoes are already worn to the limit and have to be.

(2)Replacement should becarried out at a designated ser- vice center and it is recommended that the parts made by our   company are used therein 10-20mm.

## Adjustment of Front Brake

(1)The front brake lever has a free operating   movement of 10-20mm as shown in the figure on the right side.

(2)If adjustment is needed,turn the adjusting nut near the lower side of the front hub,clockwise to re- duce and counterclockwise to increase the freeoper- ating movement of the brake lever

(3)After adjustment,the groove of the adjusting nut should be aligred with the pin of the brake arm.

Caution:

After adjustment ,check the front braking  system.The braking light should be lit up on time when the front brake is applied by gripping  the  brake  lever.

## Adjustment     of    Rear     Brake

★The vehicle shoule be sup- ported by the main stand for check.

(1)The rear brake pedal has a free operating movement of 20-30mm as   shown in the figure on theright side.

(2)To make adjustment ,turn the    rear brake adjusting nut clockwise to  reduce and counterclock wise to in- crease the free operating movement of

Rear brake pedal

Adjustment     of     Chain

★Check the chain for wear,tension and          lubrication.

(1)With the motorcycle supported by  the main stand,turn the upper and lower   portions of thechain by hand to check for   its tension to see if the sag is within the specified range of 10-20mm.

(2)When regulation is needed,loosen the axle nut and locking nut of the rear

20～30mm

the brake pedal.

wheel,then set the chain to the required

Brake cam Brake pull rod tension by turning the adjusting nut.

(3)Apply a little grease to the chain.

Caution:

At   the   nd   of  regulation,the

(3)After adjustment,the groove of   the adjusting nut should be aligred with the pin ofthe brake arm.

Caution:

After     regulation,check     the                                                  marks on the chain adjuster should

lightshould  be   litu   on   time   when                                                 engraved line on thehorizontal fork

rear   braking    system.The   braking                                                  be  in good coordination  with  the

the   rear   brake    is    applied   by                                                   al position is concerned.

stepping   down   the   brake  pedal.

22                               Rear     brake     arm           Brake arm pin

Chain adjuster(with graduations)

Rear wheel axle nut         Bear axle

Braking light switch      Adjusting nut

Braking light switch spring

## Adjustment  of  Braking  Light   Switch

★The braking light ahould be lit up on time as soon as the rearwheel is braked.If not,regulationshall be made by turning the adjusting nut.

★With the braking light switch in "ON"position,the braking light should be lit up.If not ,check should be carried out to see shether the braking lamp,circuit and switch work normally,Make replacement if needed.

Caution:

For the adjustment of the braking light switch,the brake needs to be first checked to make sure that the free  operating  movement  is  ensured  within  the  specified range.

## Battery     Vhecking

①Open the right side cover.

②Clean  away  dust  and  corro- sive from the surfaceofthe battery.

③Set the vehicle in a vertical po-

willoccur

sulfiacidetrlecausos

hurt  to  skin  and  eyes  bycontact.In

case  ofcontact  with   it,wash   it  off

for5  minutes  and  see  a  doctor  im-

## Replacement   of   Fuse

Set the ignition seitch to   "OFF"position.The  specified   fuse tube of 15A shouldbe used for replacement.

sition to see whethe the level ofthe bat- tery electrolyte is between the upperand lower mard lines.Ifit is below the lower   one,distilled water shall be added to the

mediately.

Foreign     matter    should    be  prevented  from   entering   into   the batteryduring      dismounting      and

battery.

④Seriously corroded conductor    connectorsof the battery shall be replaced

Caution:

To          dismantle          battery,

disconnent          the          negative(-)

installation.

The                 breathingpipemustbekept

unblocked.

Negative(-)                 Positive(+)

connctor                           connctor

electrode   before   thepositive(+)one,

andvice      versa      in      installation.

Ensure  against  any  contact  of  the

positive(+)electrode        with        the

vehicle    body.

Never     have     the     electrolyte

level come over the upper mark line when      adding      distilled      water.

Otherwise   overflow   and   corrosion    Lower      electro-        Upper   electrolyte

24

level    line

lyte  level  line

Open  the  left  side  cover,

remove the  fuse holder  on the  siede of the battery and replace the  fuse  tube.

If  the  new  fuse  tube  is  broken  again  as  soon  as  it  is fitted  on,it  means  that  there  is some  trouble  with  other  elec-   tric  parts.

Caution:

Do  not  use  any fuseover 15A

Be   sure   notto   wash   the  battery     when     washing     the vehicle.

## Vehicl Washing

Cleaning the vehicl regularly can slow down the color fading of its body make it easier to check if there isany damage and any oil leakage with it.

Caution:

Washing the motorcycle with over-pressurized water may cause damage to some of its components.Therefore,do not jet over-pressurized water directly on to-the following parts:

——Wheel hub

——Exhaust pipe

——Fuel tank and lower portion of cushion ——Carburetor

——Head lock and ignition switch ——Meters

(1)After pre-wiping,the vehicle shoule be washed with clean water to remove dirty residues so as to prevent

corrosion.Plastic subassemblies should be cleaned by wiping with colth orsponge soaked in neutral detergentsolution, followed by washing with clean water.

(2)Afterthe cleaned vehicle is air dried,grease the chain and run the engine at idling speed for a few minutes.

(3)Prior to driving,carefully check braking system repeatedly and repair of adjust it if necessary.

## Maintenance in Non-use Time

Storage and Maintenance

For the motorcycle to be stored for a long period of  time,attention should be paidto the prevention of moisture, sunshine and rain attack in order to protect it from un-  necessary damage.Special check-ups should be carried out on those important parts and subassemblies befor  storage.

①Change lubricating oil.

②Grease the chain.

③Drain off fuel from the fuel tank and carburtor   (forthe vehicle not to be used for over a month,the fuel in   the latter must be thoroughly drained away),turn off the   fuel cock and fill antirust solution into the fuel tank, followed by closing the tank wyth the cover:

Caution:

As  fuelis   inflammable   ,the   engine   should  be stopped before filling ordrain fuel and it is prohibited to smoke at the fuel storing,filling ordraining location.

④Take out the spark plug,fill about 15-20ml of  clean lubricating oil into the cylinder,step down the  kick lever repetitively for several times and finally fit the spark plug back on.

Attention:

The ignition switchkeymustbe set to

"OFF"position before stepping down the

kick lever.To protect the ignition system

from  damage,the  spark  plug  should  be  put

on its cap and earthed.

⑤ Dismantle the battery and put it in a shady, cool and well-ventilated place.It is suggested that the battery be charged once a month.

⑥Clean the vehicle,spray the colored part with colorfastening agent and apply antirust oil to thepart vulnerable to rust.

⑦Inflate the type as required and pad the ve- hicle up with the two wheels clear of the ground.

⑧Put the coveringover the motorcycle.

## Resumption of Service

①Remove the cover and clean the vehicle.Change the lubricating oil if the vehicl has been off service for over 4months.

②Charge the battery and remount it.

③Drain off the antirust solution from the fuel tank,followed by filling fuel therein to the required level.

④Prior todriving,test the vehicle at low speed in a safe place. Maintenance Routine Diagram

The vehicle should be under good maintenance as specified in the following table,where; "I"means:Check,cleaning,adjustment,lubrication and/or replacement are needed.

"C"means:Cleaning is needed.

"R"means:Replacement is needed. "A"means:Adjustment is needed.   "L"means:Lubrication is needed.

"*"means:This item of maintennace should be carried out at a service center.It may be also done by the user himself with reference to this manual provided he has special tools,sparts and is capable of this job.

"**"means:This item can only be carried out by the serviceman at General Accessories Corp.service center in orderto ensrue safety.

Notes:1.Maintenance should be conducted more frequently when the motorcycle drives in dusty areas.

2. When the read-outofthe odometer exceeds the maximum figures specifiedin the table,maintenance shouldbe still cycled according to the intervalofmileage statedherein.

| Frequency Item of Maintenance |  | item/Frequency | Odometer km(Note 2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1000km | 4000km | 8000km | 12000km | Remark |
| ★ | Circuit of fuel system |  |  |  | I | I |  |
| ★ | Fuel filter |  | C | C | C | C |  |
| ★ | Throttle operating sysem |  | I |  | I | I |  |
| * | Choke of carburetor |  |  | 1 | I | I |  |
|  | Airfilter element | R-yearly |  | C | C | C |  |
|  | Spark plug | R-yearly | I | I | I | R |  |
| * | Air valve gap |  | I | I | I | I |  |
|  | Air valve gap |  | I |  | I |  |  |
|  | Engine lubricating oil |  | R | One replacement every 2000km |  |  |  |
|  | Lubricating oil screen | Monthly |  | C | C | C |  |
| ★ | Tension of chain |  | A | A | A | A |  |
| ★ | ldling speed of carburetor |  |  | I | I | I |  |
|  | Driving chain | B-4 vea |  | I、Levery 500km |  |  |  |
|  | Battery |  | I | I | I | I |  |
|  | Wear of brake shoes | r-2year |  | 1 | I | I |  |
|  | Rearbraking system |  | I | I | I | I | alsofor dise style |
| ** | Braking liquid hose |  |  | 1 | I | I |  |
| *大 | Cup of braking liquid | I | I | I | I | I |  |
| ** | Braking liquid |  | One replacement every two year |  |  |  |  |
| ★大 | Front braking system |  | I |  | I | I |  |
| ★ | Rear braking light switch |  | I | I | I | I |  |
| ★ | Light changing of headlight |  | I | I | I | I |  |
|  | Clutch |  | I | I | I |  |  |
|  | Side stand |  |  | I | I | I |  |
| * | Suspension |  | I | 1 | I |  |  |
| ★ | Nuts,bolts &other fasteners |  | I | I | I | I |  |
| ** | Wheel/spokes |  |  | I | I |  |  |
| ★★ | Bearing of steering handle |  | I |  |  |  |  |

28

Remote-Controller's Function Operation And Instructions

Function operation:

●Set   acousto-optic   anti-theft:

●Remote-control   starting:

Press the button to igniteand drive the motorycycle   without   start.if   difficult,press   the   button   longer   til starting.

Press the button to stop the motorcycle in the state

Press the button The hown will sound &the turmlight of remote-control starting.

flash once.three seconds later,be in thewarning state of acousto-optic antitheft.

●Anti-theft  sensing:

In the state of anti-theft,any shock to the motor-   cycle makes the system present impedance followed with  horm sounding once &turnlight flashing once,incase of further  harassing  activities  within  the  following  three

●   Anti-rob:

In  the  state  that  the  engine  is  working,press  the button  to  stop  the  motorcycle  in  such  emergencies  as being robbed of or stolen.

●Acousto-optically     target-seeking:

Set   acousto-optic   anti-theft,therr    seconds   later,   press the button for hom and light flash so as to find the

seconds,the  system  is  to  sound  the  alam  immediately;motorcycleat  the  parking  lot.

horn ringing,turnlingtflashing and engine being lockde            Instructions  to remote-controller use.

automatically.remain  warning   state   after  the   alarm              ●The  function  of remote-control  startingonly  ap-

stopped  automatically.in  case  the  motorcycle  is  stolen,plies  to  the  motorcycles  which  is  equipped  with  auto- electricswitch  shall  flashing  and  enginebeing  lockde  matic  transmission  orable  to  electrically  start

autor\matically.

●Anti-theft  relieving:

In the state of anti-theft,press the button and the   hom will sound twice &the lightflash twice to relieve the state of anti-theft.

● Pay attention to the prevention of soaking,hard  throwing and high temperature to aviod natural loss of electrical  power.

●Do not put the keys to the remote-controller into   the lock.                                                                            29

# ELE CTRICAL DIAGRAM

|  | (1)Spark plug (2)Regulator (3)Battery (4)Fuse (5)Braking switch | (7)Start motor (8)Start button (9)Brake light (10)Taillight (i1)Meter light | B-Black Y-Yellow W-White BL.BLue |
| --- | --- | --- | --- |
| 30 | (6)Start relay | Gr-Grey | 0-Orange Br-Brown |